30/07/2026

- Exogam Project

- E.V.R.I.

- Exogam VXI Readout Interface

- Ver 1.0 (Draft version)

By: N. Karkour, D. Linget, I. Merlin (C.S.N.S.M. Orsay)

- In collaboration with:

P. Edelbruck, V. Hervier, A. Richard (IPNO France)

I. Lazarus (NPSG UK)

1.	Description of the E.V.R.I. for the GE card.	3

1.1.	Exogam Detector:	3

1.2.	DSP online data processing:	3

1.2.1.	Multiprocessing Control:	3

1.2.2.	Power Supplies Control and automatic Pole Zero Adjustment.	3

1.2.3.	On-Line Histogrammer and Datas Processing.	4

1.3.	Readout Mechanism:	4

1.4.	Electrical, and Mechanical precautions:	5

1.5.	Germanium Quad Clover AP Description	6

1.5.1.	General description.	6

1.5.2.	Detailled description.	7

1.5.3.	Pinouts of the AP Connector  for the Ge Card :	10

2.	Memory Map of the E.V.R.I. for the Ge Card	11

2.1.	VXI Configuration Registers: ( Inside the E.V.R.I.)	11

2.2.	Common Area Addresses	11

2.2.1.	Common DACs (12 bits data):	11

2.2.2.	Common Registers:	12

2.3.	Channel Area Addresses	20

2.3.1.	DAC Adjustment.	20

2.3.2.	Potentiometer  Memory Map:	21

2.3.3.	CCR Definition	21

2.3.4.	DAC Output Mode Programmation	21

2.3.5.	PARIL Setup.	22

2.4.	SRAM Area Address	24

2.5.	DSP Area Address	24

2.5.1.	IOP Registers	24

2.5.2.	Variables	25

2.5.3.	Data 32 bits	25

3.	general figures	27

3.1.	EVRI Card Dimensions.	27

3.2.	PCB_AP board connections:	28

3.2.1.	EVRI version.	28

3.2.2.	GIR V5 Version.	28

3.3.	HRGe8x Shift registers connection figure	29

3.4.	HRGe8x Dac and potentiometer connection figure	29

# Description of the E.V.R.I. for the GE card.

## Exogam Detector:

One of the solutions for electronics of the data acquisition systems in Nuclear physics experiments is to use VXI (VME eXtension bus for Instrumentation) standard electronics.

In EXOGAM we have new detectors called the Segmented Clover detector. Each detector contains 4 central contact Ge crystals, and 16 outer contact crystals with its BGO, and CSI shield. This means that electronics cannot be in one VXI card.To make things easy, all the crystals of the same type should be in the same VXI card. We have two inner contact detectors in one VXI card 16 channel VXI card for the outer contacts, and 6 Clover shields in one VXI card. These cards are called: HRGe8x (for the inner contact card) OCC16 (outer contact card), and ESS (EXOGAM Suppression Shield) for the BGO, and CSI shield. The HRGe 8x has 8 channels, OCC16  has 16 channels, and ESS has 6 shields for 6 Clovers.. card configuration logic, Readout interface logic (internal, and external), FIFO's to write the current event data in, VXI power supply filtering, Inspection lines control logic, DSP control for online data processing is implemented in a 75 mm x 366 mm pcb (Printed Circuit Board)

EVRI was accepted because it responds to the request, and the constraints asked for the HRGe8x card. The EVRI is a Readout Interface card, which is connected between the VXI back plane and the Ge Analog card, so that the 2 cards (EVRI-Analog) form a complete VXI D-size Board. This Board have programmable Input/ Output signals that are connected to the Analog cards to setup and configure the registers, and thresohlds as well as data acquisition parameters for EXOGAM.

The EVRI 's job is to read out the corrected ADC data through the PARIL (Parallel Readout Interface Logic) and writes them (or not) into the FIFO's, then these data are transferred to the ReadOut Controller Board on the VXI back plane. The Readout mechanism is completely programmable, and suitable for any acquisition system. In the case of EXOGAM  , Eurogam Readout interface for common deadtime is used.

## DSP online data processing:

### Multiprocessing Control:

This new feature is added to interconnect any assembly of VXI cards DSP's together through the VXI Local bus. This feature is completely programmable. That is by software control we can specify graphically which VXI cards can be grouped together through the Local bus, so that the DSP can have data exchange for Readout decision, and in this way we can decide in a more accurate way if the data in the FIFOs are good data or not. In another context, this decision acts as a trigger level which could be called Readout trigger, that is after the data are sent to the FIFO's the DSP can make some analysis on these data, and can interrupt the external Readout cycle if those analysis don't correspond to what the user needs.

### Power Supplies Control and automatic Pole Zero Adjustment.

Other feature for the DSP is Local control on the VXI card for Analog signal through a 16 input Analog multiplexer; there are 2 free Analog flags on the AP connector, the Analog Inspection Lines as well as the Voltage inspection line are connected to this multiplexer, and the VXI power supplies are connected too. We can, for example, control the power supplies and send flags when their level go up the references.

We can also control the Automatic Pole Zero Adjustment specific for Ge detectors or other detectors which need Pole Zero Adjustment or other Automatic Dac Adjustment (by DSP Programming). This multiplexer is connected to a local high Speed ADC, then the values are written in the DSP RAM, after which the DSP Processes the data, and send the correction to the Software on-line control through flag registers.

### On-Line Histogrammer and Datas Processing.

DSP process PARIL data and builts spectra in its external memory (Spy mode). DSP can also process datas (gain adjustment) before sending them to FIFO (Filter mode.

The EVRI has a DSP (Digital Signal Processor) which sits on the 32 bit data bus, and acts as either a spy (to process online data without modifying them) or as a filter to read the data, make some processing, and them send them to VXI backplane.

Spy mode: In this mode the DSP just collects the data at the same time, as they go onto the VXI backplane, during an event and make processing on these data, and puts the result in its Memory. This can help the user to check on the results inside each card during an experiment to make sure that the card setup is done good. The DSP DOES NOT make any modification on the good data .

Filter mode: The EVRI can be programmed by software to make the DSP read the data without sending it to the VXI backplane, and then the DSP, automatically, makes data processing, and sends back the results to EVRI readout interface which sends it back to the VXI backplane. In this mode the DSP can make any kind of filtering on the data, and even keeps the raw results, and add more processed data inside the same event. This method can help the user to get more information from each event in addition to the raw data.

## Readout Mechanism:

The Readout Mechanism will be divided into two cycles: Internal, and External Readout cycles. The Internal cycle is the cycle which takes place for data transfer between the Analog card, and the EVRI. While the External Readout cycle is the interface between the G.I.R, and the Readout Controller Board. In EXOGAM, each PARIL's chips will be used on the Analogue card to put the ADC data from two channels into registers (32 bit wide) to be transferred to the EVRI into the FIFO's. Each PARIL has a 2 direct 16 bits data bus connected to the 2 channel's DPPC.

In the External Readout cycle the Eurogam Readout interface will be used. We will upgrade later the Readout interface to several Readout mechanisms due to programmable devices (e.g. SSBLT or MBLT 64 transfer etc.).

In EXOGAM  the EVRI uses as in Eurogam the same VXI lines dedicated for the Digital, and Analog multiplexing lines, Fast trigger, and Validation signals, etc...

First, When the CFD of a Ge channel fires and when the Sumbus signal exceeds the sumbus threshold in the MASTER trigger, this latter sends a Fast Trigger signal followed by a Validation signal (with the event no. on the VXI local bus) as a confirmation that the present event (which is already running) is a good event to be coded, and the data in the ADCs are to be read out or transferred into the FIFO.

When it receives Validation the crate ReadOut COntroller asserts the AS* (Address Strobe) signal onto the VXI back plane with the event no. on the VME address bus, and waits for the VXI boards that participate in this event by asserting or not BLTACK* (BLTACK* is an open collector signal). At that time the Readout interface inside each EVRI maintains BLTACK* if Validation Acknowledge signal from the Local Trigger(s) is asserted, and it waits until the ADC data are sent out to the FIFO. The ROCO reads all the data from all the VXI cards by daisy chain. For more details about how the EVRI builds its interface refer to the Readout document, concerning both the Internal, and the External Readout mechanism.

## Electrical, and Mechanical precautions:

This card is specifically designed for low noise Analog data acquisition systems. It takes care of the EMC problems. It already contains several elements which worked before in other similar cards. Data transfer is 20 Mbytes/sec 5 Mwords/sec, and in Nuclear physics data acquisition systems the shaping time and the coding time take highest percentage of the total channel dead time.

The aim of this Board is to have Readout interface completely programmable for different Readout mechanisms, and which can be suitable for different data acquisitions. That is it can be adapted to either fast or slow data transfers. The Analog engineer need not be anxious about the Readout part of his Board. He needs only to design his Board up to digital data, and then he has to choose the Readout system to be adapted for his experiment, so that the Readout interface can be built.

EMC problems are very dangerous to sensitive Analog signals, especially in Nuclear physics data acquisitions. EVRI has the middle column of the 168 pin AP connectors (as shown previously) mostly grounded. Extra flat connectors will be used to minimise the electromagnetic noise penetrations. 16 MHz VME Clock will be  used on the Board for the DSP. Slow asynchronous data transfer from the FIFO's. (see the PCB_AP connections diagram.). For example the 2 Analog Inspection Lines are surrounded with ground pins. Power Supply chokes, and OSCON capacitors are used to filter the VXI power supplies.

The interconnection between the Analog card and EVRI can be shown in the figures below showing the top, bottom, and face views. High ground precautions is taken on the top of the Mother Board by using a Copper Barrel shield over the three connectors, and on the bottom 2 small copper joints are fixed tightly as shown in the figure (EVRI- Analog). On the top side EMC joints are to be glued on those barrels so that they have good connections to the external VXI Board shield. The two diagrams, one with the mechanical details of the Copper Barrel, and one shows how the EVRI will be mounted data can show us clearly how the system is implemented. An extractor tool is built to extract the PCB_AP connector (300 pins) out of both the EVRI, and the Analog card. A special tool exists to mount, and unmount the EVRI, and the Analog card together with the PCB_AP without having any pressure on the SMD components, just on the surrounding of both PCB's (see the mechanical dimensions, & the isometric view figures for more details).

A new mechanic has been adopted to protect cards from high temperature and EM perturbations. The card is now fixed on 2 rails (upper and lower) and the whole is inserted in the crate slot. This has induced the decrease of the PCB dimensions, but the manipulation of the card during prototype tests is easier and safer. The rails are pierced with 450 holes each, increasing the airflow area about 50%. 2 shields are fixed by 36 screws in the rails, avoiding the alignment problem causing by the passage through the PCB in the old mechanic. EMC performances are above 90dB

## Germanium Quad Clover AP Description

The EVRI is connected to the Analog card through three connectors (2 x 50= 100 pins). These connectors are called Analog_P (AP) connectors. The connections between the EVRI, and the Analog card is made through a PCB (called the PCB_AP). The middle column of this connector is mostly connected to the ground for EMC precautions (for more details refer to the Electrical Precautions). These connectors are called AP1, AP2, & AP3 ( from top to bottom). All three connectors are fully compatible so that the same PCB_AP is used. For example the same power supply pins are connected through a routing shape. Table 1 describes the three AP connector pins, and their locations.

### General description.

· AP1:

This connector contains the following signals:

a. 76 Input/ Output pins (called Reg_Io) which are fully programmable pins of a XILINX FPGA (LCA decode1).

b. 7 bits of a latched address bus (ADD 1 to ADD 7).

c. Filtered -2 volts VXI power supply.

d. 1 Analog Flag free signal which is connected to an analog multiplexer inside the EVRI, and which can be digitised by the DSP program, plus two digital free I/O signals(RDFLAG1,2).

· AP2:

The AP2 contains the following signals:

a. The higher 16 bit of  a latched address bus (ADD 8 to ADD 23).

b. There is also the Analog, Digital, and Voltage Inspection Lines which come from the Analog card.

c. the lower 16 bits of the bidirectional data bus.

d. 16 input/output reprogrammable pins (called Reg_Io) from XILINX FPGA (LCA Decode2).

e. 18 input/output pins from the ReadOut LCA  (READ 0 to READ 17 except READ 6,8,10,11,14 which are buffered output signals to the Analog card.

f. 4 TTL signals which come from the VXI back plane as ECL signals (they are the Trigger Event numbers in EXOGAM ),

g. the VME 16 MHz Clock, and filtered  +/- 12 volts VXI power supply.

h. 1 Analog Flag free signal which is connected to an analog multiplexer inside the EVRI, and which can be digitised by the DSP program, and two digital free I/O signals (RDFLAG3 ,4).

· AP3:

The AP3 connector contains the following signals:

a. The higher 16 bits of the data bus (DATA 16 to DATA 31).

b. 5 input/output signals from the ReadOut LCA (READ 18 to READ 22).

c. The Differential VXI line Starx +/- , and 2 TTL Trigger lines (which is the Fast Trigger, Validation, and Inhibit signals respectively). They are the EXOGAM MASTER Trigger signals.

d. Moreover there are 49 INPUT/output pins from LCA Decode2 (called Reg_IO).

e. 4 bits which are connected to the highest nibble in the FIFO, they are the event no. latched in the Analog Board.

f. 3 TTL signals which can be used as flags for the DSP

### Detailled description.

#### AP1 :

- DAC Data : serial data signal to load the DACs, this signal is common for all the Dacs.

- DAC Clk : clock signal to load the DACs, this signal is common for all the Dacs.

- DAC Select : DAC Chips Select signal of the Ge Analog card.

- Pot Select : Potentiometer Chip Select Signal of the Ge Analog card.

- DCS n  : Chip Select signal to load the DACs, there is one signal per Channel (n =0 to 7, is the number of channel).

- DCS Common : Common DAC chip select signal of the Ge Analog card

- MUX data : Serial data signal to load the Multiplexers, this signal is common for all the Multiplexers.MUX Clk : Clock signal to load the Multiplexers, this signal is common for all the Multiplexers.

- MuxStrobe m : Multiplexeur number m strobe signal to load the Multiplexers (m=0 to 2)

- CCR Data : serial data signal to load the Registers, this signal is common for all the Registers.

- CCR Clk : clock signal to load the Registers, this signal is common for all the Registers.

- CCR Strobe n : Channel Control Register Strobe Signal there is one signal per Channel. (n =0 to 7, is the number of channel).

- Synchro : Signal sent by Master Trigger to simulate CFD to all channel in test mode.

- Go/Stop* : buffered Stop*/Go signal from the MASTER Trigger (on the VXI back plane).

- Latch Scaler : This signal is sent to every PARIL to latch counter value

- ADD[10..2] : this is the lowest byte of the VME Address bus, it is enabled by the EVRI decoding.

- LED: display signal of LED indicator driven by an open collector from E.V.R.I. to the front panel :

- Temp Led : ON if Over or under  normal temperature.

- Error Led : ON if Fifo Error.

- Done Led : ON if all LCAs are programmed.

- AFLAG 1 :  free signal which is connected to an analog multiplexer inside the EVRI, and which can be digitised by the DSP program

#### AP2 :

- PARIL LI 1,2 : Digital Inspection Lines common for all PARIL (Used mainly in Debug phase).

- StartReadout_Cor : This signal is the Start_Readout of the last PARIL inside the card, which is the back edge of the last Busy signal which corresponds to a correlated event.

- Last Pass_Cor : Renout of the last PARIL for correlated events, indicates end of readout cycle.

- Dtready 0 to 3 : data ready given by the PARIL when a data is ready to be latched on the bus.

- AIm : Analogue Inspection Lines, m is the inspection line indicator (m=1,2).

- DIm : Digital Inspection Lines, m is the inspection line indicator(m=1,2).

- VI : Voltage Inspection Line.

- DATA[15..0] : these bits are the 16 LSB E.V.R.I. Data bus. This bus has 2 functions : it is the VME data bus during read-write operation of the PARILs and EVRI DSP data bus during Readout.

- LT_Reset cor : This signal indicates the end of the event processing. It goes to PARIL which sends it to the correlated Local Triggers that were read out.

- Dtack : buffered data acknowledge signal for each word of the PARILs in readout cycle, common for all the PARILs.

- StartReadout_Sin : This signal is the Start_Readout_Singles_Out of the Last PARIL inside the card, which is OR of the daisy chained Start_Readout_singles of all PARILs on the card. This signal is asserted when the DPPC data are transfered to PARIL. It is the OR of all Start_Readout_singles, because EVRI MUST know when a singles event is ready to be read out, and in this way EVRI asserts back Latch_Singles_Event which is in the next paragraph.

- Latch_single_evts : This signal is asserted by EVRI when:

- the first Start_Readout_Singles is asserted OR

- at the end of a any current Readout cycle during which a Start_Readout_Singles is asserted. This means that there is a new singles event to be readout and hence Latch_Singles_event is asserted again.

- Last Pass_Sin : Renout of the last PARIL for Single events, indicates end of readout cycle.

- LT_Reset singles : This signal indicates the end of the event processing. It goes to PARIL which sends it to the singles Local Triggers that were read out.

- Global Reset : buffered global reset signal.

- OVER Temp* :

- RD FLAG 2 to 4 : three digital free I/O signals

#### AP3 :

- DATA[31..16] : these bits are the 16 MSB E.V.R.I. Data bus. This bus has 2 functions : it is the VME data bus during read-write operation of the PARILs and EVRI DSP data bus during Readout.

- DProm_Read:  serial PROM Data to be read out of the Ge Analog card.

- CSProm : serial PROM Chip Select signal of the Ge Analog card.

- R/W : buffered VME read-write signal.

- Last Valack : this signal is the Validation Acknowledge given by the last (Daisy chain) Local Trigger.

- Renin_Sin : Start readout cycle of PARIL for single events, this signal is sent to the first PARIL.

- Renin_Cor : Start readout cycle of PARIL for correlated events, this signal is sent to the first PARIL.

- Inhibit* : buffered Inhibit signal from the MASTER Trigger (on the VXI back plane). It is used by Local Triggers only when singles are disabled.

- Fast Trigger : buffered Fast Trigger signal ( positive TTL ) from the MASTER Trigger (starX +/- on the VXI back plane).

- Validation* : buffered Validation signal from the MASTER Trigger (on the VXI back plane).

- Beam RF : Beam signal

- LINK_D [3..0]: Data bus for the DSP Link, Used for the Single data transferts.

- LINK_CLK : clock signal for the DSP Link, Used for the Single data transferts.

- LINK_ACK : acknowledge signal for the DSP Link, Used for the Single data transferts.

- Prom_CCL_Din : serial PROM and CCL Data write to the Ge Analog card

- Prom CCLClk : serial PROM and Clock signal of the Ge Analog card.

- CCL_CS : CCL Chip Select signal for the configuration.

- Cs_PARIL 0 to 3 : PARIL Chip Select signal for the registers configuration, one per PARIL.

- PARIL_Strobe : buffered data strobe signal for the configuration of the PARILs, common for all the PARILs.

- Sel_Odd/Even : Select the odd or even channel inside the PARIL selected by its CS

- SystemClk : buffered Clock signal (up to 50 Mhz).

- DSPFLAG 1to 2 : TTL signals which can be used as flags for the DSP

#### Power distribution :

- + / - 5 V 	: 12 pins each.

- -2 V 		: 6 pins.

- + / - 12 V 	: 3 pins each.

- + / - 24 V 	: 3 pins each.

### Pinouts of the AP Connector  for the Ge Card :

|  |  |  | AP1 Connector |  |  |  |  |  | AP2 Connector |  |  |  |  |  | AP3 Connector |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PIN NO. |  |  | ROW A |  | ROW B |  | ROW A |  |  | ROW B |  | ROW A |  |  | ROW B |  | PIN NO. |  |  |
| 1 |  | +5 V |  |  | +5v |  | +5 V |  |  | +5 V |  | +5v |  | +5v |  |  | 1 |  |  |  |
| 2 |  | Dac Data |  |  | GND |  | Paril LI1 |  |  | GND |  | DATA 16 |  | GND |  |  | 2 |  |  |  |
| 3 |  | Dac Clk |  |  | Temp LED |  | Paril LI2 |  |  | OverTemp* |  | DATA 17 |  | DSPFLAG1 |  |  | 3 |  |  |  |
| 4 |  | DAC Select |  |  | GND |  | Start_Readout_corr |  |  | GND |  | DATA 18 |  | GND |  |  | 4 |  |  |  |
| 5 |  | Pot Select |  |  | error LED |  | Read_IO |  |  | RDFLAG2 |  | DATA 19 |  | DSPFLAG2 |  |  | 5 |  |  |  |
| 6 |  | DCS 0 |  |  | GND |  | LastPass_Correlated |  |  | GND |  | DATA 20 |  | GND |  |  | 6 |  |  |  |
| 7 |  | DCS 1 |  |  | DONE LED |  | Dtready0 |  |  | RDFLAG3 |  | DATA 21 |  | DSPFLAG3 |  |  | 7 |  |  |  |
| 8 |  | DCS 2 |  |  | GND |  | Dtready1 |  |  | GND |  | DATA 22 |  | GND |  |  | 8 |  |  |  |
| 9 |  | DCS 3 |  |  | AFLAG1 |  | GND |  |  | RDFLAG4 |  | DATA 23 |  | AFLAG2 |  |  | 9 |  |  |  |
| 10 |  | DCS 4 |  |  | GND |  | AI 1 |  |  | GND |  | DATA 24 |  | GND |  |  | 10 |  |  |  |
| 11 |  | DCS 5 |  |  | GND |  | GND |  |  | GND |  | DATA 25 |  | GND |  |  | 11 |  |  |  |
| 12 |  | DCS 6 |  |  | GND |  | AI 2 |  |  | GND |  | DATA 26 |  | GND |  |  | 12 |  |  |  |
| 13 |  | DCS 7 |  |  | GND |  | GND |  |  | GND |  | DATA 27 |  | GND |  |  | 13 |  |  |  |
| 14 |  | DCS Common |  |  | GND |  | DI 1 |  |  | GND |  | DATA 28 |  | GND |  |  | 14 |  |  |  |
| 15 |  | -5.2 V |  |  | -5.2 V |  | -5.2 V |  |  | -5.2 V |  | -5.2 V |  | -5.2 V |  |  | 15 |  |  |  |
| 16 |  | Mux Data |  |  | GND |  | DI 2 |  |  | GND |  | DATA 29 |  | GND |  |  | 16 |  |  |  |
| 17 |  | Mux Clk |  |  | GND |  | GND |  |  | GND |  | DATA 30 |  | GND |  |  | 17 |  |  |  |
| 18 |  | MuxStrobe0 |  |  | GND |  | VI |  |  | GND |  | DATA 31 |  | GND |  |  | 18 |  |  |  |
| 19 |  | MuxStrobe1 |  |  | GND |  | DATA 0 |  |  | GND |  | DPromRD |  | GND |  |  | 19 |  |  |  |
| 20 |  | MuxStrobe2 |  |  | GND |  | DATA 1 |  |  | GND |  | PromCS |  | GND |  |  | 20 |  |  |  |
| 21 |  | CCR Data |  |  | GND |  | DATA 2 |  |  | GND |  | ReadWrite |  | GND |  |  | 21 |  |  |  |
| 22 |  | CCR Clk |  |  | GND |  | DATA 3 |  |  | GND |  | Last Valack |  | GND |  |  | 22 |  |  |  |
| 23 |  | CCRStrobe0 |  |  | GND |  | DATA 4 |  |  | GND |  | Renin_Singles |  | GND |  |  | 23 |  |  |  |
| 24 |  | CCRStrobe1 |  |  | GND |  | DATA 5 |  |  | GND |  | Renin_Corr |  | GND |  |  | 24 |  |  |  |
| 25 |  | -5.2 V |  |  | -5.2 V |  | -5.2 V |  |  | -5.2 V |  | -5.2 V |  | -5.2 V |  |  | 25 |  |  |  |
| 26 |  | CCRStrobe2 |  |  | GND |  | DATA 6 |  |  | GND |  | Inhibit* |  | GND |  |  | 26 |  |  |  |
| 27 |  | CCRStrobe3 |  |  | GND |  | DATA 7 |  |  | GND |  | FastTrigger |  | GND |  |  | 27 |  |  |  |
| 28 |  | CCRStrobe4 |  |  | GND |  | Reg_IO |  |  | GND |  | Validation* |  | GND |  |  | 28 |  |  |  |
| 29 |  | CCRStrobe5 |  |  | GND |  | DATA 8 |  |  | GND |  | BeamRF |  | GND |  |  | 29 |  |  |  |
| 30 |  | CCRStrobe6 |  |  | GND |  | DATA 9 |  |  | GND |  | LINK_D0 |  | GND |  |  | 30 |  |  |  |
| 31 |  | CCRStrobe7 |  |  | GND |  | DATA 10 |  |  | GND |  | LINK_D1 |  | GND |  |  | 31 |  |  |  |
| 32 |  | Synchro |  |  | GND |  | DATA 11 |  |  | GND |  | LINK_D2 |  | GND |  |  | 32 |  |  |  |
| 33 |  | Go*/Stop |  |  | GND |  | DATA 12 |  |  | GND |  | LINK_D3 |  | GND |  |  | 33 |  |  |  |
| 34 |  | Latch_Scaler |  |  | GND |  | DATA 13 |  |  | GND |  | LINK_CLK |  | GND |  |  | 34 |  |  |  |
| 35 |  | Reg_Io |  |  | GND |  | DATA 14 |  |  | GND |  | LINK_ACK |  | GND |  |  | 35 |  |  |  |
| 36 |  | Reg_Io |  |  | GND |  | DATA 15 |  |  | GND |  | Prom_CCL_Din |  | GND |  |  | 36 |  |  |  |
| 37 |  | ADD2 |  |  | GND |  | Dtready2 |  |  | GND |  | Prom_CCL_Clk |  | GND |  |  | 37 |  |  |  |
| 38 |  | ADD3 |  |  | GND |  | Dtready3 |  |  | GND |  | CCL_CS |  | GND |  |  | 38 |  |  |  |
| 39 |  | ADD4 |  |  | GND |  | LT_Reset_corr |  |  | GND |  | PARILCS0 |  | GND |  |  | 39 |  |  |  |
| 40 |  | ADD5 |  |  | GND |  | Dtack |  |  | GND |  | PARILCS1 |  | GND |  |  | 40 |  |  |  |
| 41 |  | ADD6 |  |  | GND |  | Start_Readout_Singles |  |  | GND |  | PARILCS2 |  | GND |  |  | 41 |  |  |  |
| 42 |  | ADD7 |  |  | GND |  | Latch_Singles_event |  |  | GND |  | PARILCS3 |  | GND |  |  | 42 |  |  |  |
| 43 |  | ADD 8 |  |  | GND |  | LastPass_Singles |  |  | GND |  | Paril Strobe |  | GND |  |  | 43 |  |  |  |
| 44 |  | ADD 9 |  |  | GND |  | LT_Reset Singles |  |  | GND |  | SystemCLK |  | GND |  |  | 44 |  |  |  |
| 45 |  | ADD 10 |  |  | GND |  | GlobalReset |  |  | GND |  | Sel_Odd/Even |  | GND |  |  | 45 |  |  |  |
| 46 |  | +5 V |  |  | +5 V |  | +5 V |  |  | +5 V |  | +5 V |  | +5 V |  |  | 46 |  |  |  |
| 47 |  | -2 V |  |  | GND |  | +12 V |  |  | GND |  | +24 V |  | GND |  |  | 47 |  |  |  |
| 48 |  | -2 V |  |  | -2 V |  | +12 V |  |  | +12 V |  | +24 V |  | +24 V |  |  | 48 |  |  |  |
| 49 |  | -2 V |  |  | GND |  | -12 V |  |  | GND |  | -24 V |  | GND |  |  | 49 |  |  |  |
| 50 |  | -2 V |  |  | -2 V |  | -12 V |  |  | -12 V |  | -24 V |  | -24 V |  |  | 50 |  |  |  |

# Memory Map of the E.V.R.I. for the Ge Card

| VME Address | Function |
| --- | --- |
| 0xFFFFFF80 to 0xFFFFFF8A | VXI Configuration Registers. (See §2.1) |
| 0x0 to 0x3FE | Common Area Address (See § 2.2) |
| 0x1000 to 0x82FC | Channel Area Address (See §2.3) |
| 0x200000 to 03FFFFC | SRAM Area Address (See § 2.4) |
| 0x400000 to 0x4BFFFC | DSP Area Address (See § 2.5) |

## VXI Configuration Registers: ( Inside the E.V.R.I.)

The VXI base address is calculated as follow : 0xFFFFFF80 - (n * 0x40)  where n is the number of cards between the Resource Manager and the Target card (n = 0 to 11).

| VXI Base Address (slot 1) | Description | DATA Format |
| --- | --- | --- |
| 0xFFFFFF80 | In Read Mode this is the Man. ID. Device Class and Address Space | d15, d14 = 0x3, d13, d12=01(A16/A32) or 00 (A16/A24), d11, d8 =1111for IN2P3 d7,d0=Pr (0x5A) for IN2P3 from PROM |
| 0xFFFFFF80 | In Write mode the R.M. Writes the Logic Address of the VXI card in A13,A6 | D7 to D0 is the logical Address |
| 0xFFFFFF82 | Read Only Register for the Device type and the model code | d15,d12 = 0x 7 (A24) or 0xf (A32) d11,d8=model code d7 to d0 from PROM |
| 0xFFFFFF84 | In Read mode: Status Register | d15 to d0. |
| 0xFFFFFF84 | Write mode: Control Register | d15 to d0 |
| 0xFFFFFF86 | Read/Write Offset Register | d15,d8 for A24, d15,d0 for A32 |
| 0xFFFFFF88 | Read Only: Serial no. | Read 8 bits only from PROM d7 to d0 |
| 0xFFFFFF8a | Read Only: Modification Level | Read 8 bits only from PROM d7 to d0 |

## Common Area Addresses

| VME Address | Bus Width | Rd/ Wr | Function |
| --- | --- | --- | --- |
| 0x0 to 0x6 | 16 | w | Common DACs (see § 2.2.1) |
| 0x10 to 0x3FE | 16 | w | Common Registers (see § 2.2.2) |

### Common DACs (12 bits data):

| VME Address | Bus Width | Read/ Write | Function |
| --- | --- | --- | --- |
| 0x0 | 16 | w | Valid_Threshold |
| 0x2 | 16 | w | Addback_Wind_Threshold |
| 0x4 | 16 | w | Not allocated |
| 0x6 | 16 | w | CFD Width Adjustment |

- Max 536 quadruple DAC 12 bits, buffered outputs, MSB sent first.

| A1 | A0 | C1 | C0 | D11…D0 |
| --- | --- | --- | --- | --- |
| Decoded Address 0-3 |  | 1 | 1 | DAC Value |
|  |  | Load input register with D11-D0 value, updated output |  |  |

### Common Registers:

| Global Addresses | Bus Width | Read/ Write | Function |
| --- | --- | --- | --- |
| 0x10 | 16 | r/w | GIR Module Control Register (MCR) (see §2.2.2.1) |
| 0x12 | 16 | w | Ge Inner Control Register (GICR) (see §) |
| 0x14 | 16 | w | DSP DAC 12 bits |
| 0x16 | 16 | w | DSP Potar 8 bits |
| 0x20 to 0x32 | 16 | r/w | Inspection Lines(see §2.2.2.3) |
| 0x40 to 0x5E | 16 | r/w | Temperature Control Register (see § 2.2.2.4) |
| 0x100--0x1FE | 16 | r/w | Analogue Card EEPROM (see § 2.2.2.5) |
| 0x300--0x3FE | 16/32 | r/w | E.V.R.I. Readout Setup (see § 2.2.2.6) |

#### GIR  Module Control Register

VME base Address :  0x10

| Bit Number | Meaning when = 0 | Meaning when = 1 |
| --- | --- | --- |
| 0 | Not Last Card | Last Card (use terminators) |
| 1 | Reset FIFO Disable | FIFO Reset |
| 2 | Reset DSP Disable | DSP Reset |
| 3 | Reset GIR Disable | GIR Reset |
| 4 to 15 | Not Allocated | Not Allocated |

#### Ge Inner Control Register

VME base Address :  0x12

| Bit Number | Meaning when = 0 | Meaning when = 1 |
| --- | --- | --- |
| 0 | Mark Pre Pileup Events | Reject Pre Pileup Events |
| 1 | Mark Veto | Reject Veto |
| 2 | Not Allocated | Not Allocated |
| 3 | Single Event Disable | Single Event Enable |
| 4 | Use FT to Stop TAC | Use Beam to Stop TAC |
| 5 | Use Raw Multiplicity | Use Clean Multiplicity |
| 6 | Write Protect Prom on | Write Protect Prom off |
| 7 | Enable mode channel | Enable mode clover |
| 8 to 10 | CCL code 0 | see §2.3.2.1.1 |
| 13 to 15 | Not Allocated | Not Allocated |

##### CCL Code  : bit 8 to 10 of GICR register

| Bit | Mode |  |
| --- | --- | --- |
| 000 | Power up | Each CFD triggers its own channel |
| 001 | Addback | Each CFD triggers its own channel starts all 4 channels for low energy signals |
| 010 | Mode 2 | CFD 1 triggers channels 0,1,2,3 and CFD 5 triggers channels 4,5,6,7 |
| 011 | Mode 3 | CFD 0 triggers channels 0,1,2,3 and CFD 4 triggers channels 4,5,6,7 |
| 100 | Mode 4 | CFD 0  channels 0,1; CFD 2  channels 2,3; CFD 4  channels 4,5; CFD 6  channels 6,7; |

#### Inspection lines:

| 0x20 | 16 | r/w | Digital Inspection 1 channel select (see § 2.2.2.3.1) |
| --- | --- | --- | --- |
| 0x22 | 16 | r/w | Digital Inspection 1 parameter Select |
| 0x24 | 16 | r/w | Digital Inspection 2 channel select |
| 0x26 | 16 | r/w | Digital Inspection 2 parameter Select |
| 0x28 | 16 | r/w | Analog Inspection 1 channel select (see § 2.2.2.3.2) |
| 0x2A | 16 | r/w | Analog Inspection 1 parameter Select |
| 0x2C | 16 | r/w | Analog Inspection 2 channel select |
| 0x2E | 16 | r/w | Analog Inspection 2 parameter Select |
| 0x30 | 16 | r/w | Voltage Inspection channel select (see § 2.2.2.3.3) |
| 0x32 | 16 | r/w | Voltage Inspection parameter Select |

##### Digital Inspection Lines.

- To Disable the Digital inspection Line 1 write 0x00 at address 0x20.

- To Disable the Digital inspection Line 2 write 0x00 at address 0x24.

| Parameters for line 1 & 2 | Data Value for line Address vme base address line1 = 0x20 vme base address line2 = 0x24 | Data Value for Parameter Address vme base address parl1 = 0x22 vme base address parl2 = 0x26 |
| --- | --- | --- |
| Disable | 0x0 | xx |
| CFD Direct Channel 0 | 0x1 | 0x0 |
| CFD Delayed Channel 0 | 0x1 | 0x1 |
| Start_LT in Channel 0 | 0x1 | 0x2 |
| FT Sample point Channel 0 | 0x1 | 0x3 |
| Val Sample Point Channel 0 | 0x1 | 0x4 |
| PDS Gate Channel 0 | 0x1 | 0x5 |
| Reset (TAC) Channel 0 | 0x1 | 0x6 |
| Prepileup in view Channel 0 | for line 1 0x1 | 0x7 |
| LT Reset Out Channel 0 | for line 2 0x1 | 0x7 |
| Same Values Channel 1 | 0x2 | see upper |
| Same Values Channel 2 | 0x3 | see upper |
| Same Values Channel 3 | 0x4 | see upper |
| Same Values Channel 4 | 0x5 | see upper |
| Same Values Channel 5 | 0x6 | see upper |
| Same Values Channel 6 | 0x7 | see upper |
| Same Values Channel 7 | 0x8 | see upper |

| E.V.R.I. Global Parameters | GIR Control Lines | line Data Value | Parameter Data Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Disable |  | 0x20 | 0x000 |  |  |  |
| TRIGGER SIGNALS. |  |  |  |  |  |  |
| Fast_Trigger | 0x001 | 0x20 | 0x01 |  |  |  |
| Inhibit_Action (M.T.) | 0x002 | 0x20 | 0x02 |  |  |  |
| Validation | 0x003 | 0x20 | 0x03 |  |  |  |
| Event Reject | 0x004 | 0x20 | 0x04 |  |  |  |
| Beam Pulse | 0x005 | 0x20 | 0x05 |  |  |  |
| CORROLATED READOUT PARIL to EVRI |  |  |  |  |  |  |
| OrValack | 0x06 | 0x20 | 0x06 |  |  |  |
| Conversion DTime (coding) | 0x07 | 0x20 | 0x07 |  |  |  |
| StartReadout Corrolated | 0x08 | 0x20 | 0x08 |  |  |  |
| Paril Renin Corrolated | 0x09 | 0x20 | 0x09 |  |  |  |
| DTready0 | 0x0A | 0x20 | 0x0A |  |  |  |
| DTready1 | 0x0B | 0x20 | 0x0B |  |  |  |
| DTready2 | 0x0C | 0x20 | 0x0C |  |  |  |
| DTready3 | 0x0D | 0x20 | 0x0D |  |  |  |
| DTack_buf | 0x0E | 0x20 | 0x0E |  |  |  |
| Paril Renout Corrolated | 0x0F | 0x20 | 0x0F |  |  |  |
| CLKWF | 0x10 | 0x20 | 0x10 |  |  |  |
| ENWF | 0x11 | 0x20 | 0x11 |  |  |  |
| SINGLE READOUT PARIL to EVRI |  |  |  |  |  |  |
| StartReadout Singles | 0x12 | 0x20 | 0x12 |  |  |  |
| Paril Renin Singles | 0x13 | 0x20 | 0x13 |  |  |  |
| Latch singles Event | 0x14 | 0x20 | 0x14 |  |  |  |
| Paril Renout Singles | 0x15 | 0x20 | 0x15 |  |  |  |
| CORROLATED VXI READOUT |  |  |  |  | 0x20 | 0x015 |
| AS* (VME) | 0x16 | 0x20 | 0x16 |  |  |  |
| DS (DS0* and DS1*) (VME) | 0x17 | 0x20 | 0x17 |  |  |  |
| R/W* (VME) | 0x18 | 0x20 | 0x18 |  |  |  |
| ModRoco | 0x19 | 0x20 | 0x19 |  |  |  |
| CardRenin *(From VRE) | 0x1A | 0x20 | 0x1A |  |  |  |
| BLTACK (Card) | 0x1B | 0x20 | 0x1B |  |  |  |
| BLTACK (Bus) | 0x1C | 0x20 | 0x1C |  |  |  |
| FDTACK | 0x1D | 0x20 | 0x1D |  |  |  |
| DTACK* (VME) | 0x1E | 0x20 | 0x1E |  |  |  |
| Oe_EVRI Counter | 0x1F | 0x20 | 0x1F |  |  |  |
| CardRenout (Bus Renout) | 0x20 | 0x20 | 0x20 |  |  |  |
| LT_Reset* | 0x21 | 0x20 | 0x21 |  |  |  |
| ClkB_Fifo | 0x22 | 0x20 | 0x22 |  |  |  |
| ENR-F1 | 0x23 | 0x20 | 0x23 |  |  |  |
| Or-Fifo | 0x24 | 0x20 | 0x24 |  |  |  |
| Reset-Fifo | 0x25 | 0x20 | 0x25 |  |  |  |
| Sysclk | 0x26 | 0x20 | 0x26 |  |  |  |
| DSP INTERFACE |  |  |  |  |  |  |
| en_DMA_Paril | 0x27 | 0x20 | 0x27 |  |  |  |
| DMA-Req | 0x28 | 0x20 | 0x28 |  |  |  |
| DMA-Grant | 0x29 | 0x20 | 0x29 |  |  |  |
| Link_Start | 0x2A | 0x20 | 0x2A |  |  |  |
| Link5_Ack | 0x2B | 0x20 | 0x2B |  |  |  |
| Link5_Clk | 0x2C | 0x20 | 0x2C |  |  |  |
| Link5_Dat0 | 0x2D | 0x20 | 0x2D |  |  |  |
| Link5_Dat1 | 0x2E | 0x20 | 0x2E |  |  |  |
| Link5_Dat2 | 0x2F | 0x20 | 0x2F |  |  |  |
| Link5_Dat3 | 0x30 | 0x20 | 0x30 |  |  |  |
| OE_Link5_out | 0x31 | 0x20 | 0x31 |  |  |  |
| OE_DSP_Run | 0x32 | 0x20 | 0x32 |  |  |  |
| Readout-Evt-Fault-M1 | 0x33 | 0x20 | 0x33 |  |  |  |
| Readout-Evt-Fault | 0x34 | 0x20 | 0x34 |  |  |  |
| Readout-Evt-Match | 0x35 | 0x20 | 0x35 |  |  |  |
| Paril-LI1 | 0x36 | 0x20 | 0x36 |  |  |  |
| Paril-LI2 | 0x37 | 0x20 | 0x37 |  |  |  |
| Not Used (DI Disable) | Disable | valus not = 0x20 | xxxx |  |  |  |

##### Analog inspection lines:

- To Disable the Analog inspection Line 1 write 0x00 at address 0x28.

- To Disable the Analog inspection Line 2 write 0x00 at address 0x2C.

| Parameters for line 1 & 2 |  | Data Value for line Address vme address li1 = 0x28 vme address li2 = 0x2C | Data Value for Param Address vme base address para1 = 0x2A vme base address para2 = 0x2E |
| --- | --- | --- | --- |
| Disable |  | 0x0 | xx |
| Preamplifier In Channel 0 | 0 -10V | 0x1 | 00 |
| 6 MeV out Channel 0 | 0 +10V | 0x1 | 01 |
| 20 MeV out Channel 0 | 0 +10V | 0x1 | 02 |
| PZ / BLR monitor Channel 0 |  | 0x1 | 03 |
| 6 MeV PDS Channel 0 | 0 +10V | 0x1 | 04 |
| TAC out Channel 0 | 0 +5V | 0x1 | 05 |
| CFD in (TFA Out) Channel 0 |  | 0x1 | 06 |
| DPPC In |  | 0x1 | 07 |
| Same Values Channel 1 |  | 0x2 | see upper |
| Same Values Channel 2 |  | 0x3 | see upper |
| Same Values Channel 3 |  | 0x4 | see upper |
| Same Values Channel 4 |  | 0x5 | see upper |
| Same Values Channel 5 |  | 0x6 | see upper |
| Same Values Channel 6 |  | 0x7 | see upper |
| Same Values Channel 7 |  | 0x8 | see upper |

##### Voltage inspection line: (only common lines)

- To Disable the Voltage Inspection line : write 0x00 at address 0x30.

| Global parameters | Data Value for line Address | Data Value for Parameter Address |
| --- | --- | --- |
| Vtemp Lower side | 0x1 | 0x0 |
| Vtemp Upper side | 0x1 | 0x1 |
| Vp 20 | 0x1 | 0x2 |
| Vp 15 | 0x1 | 0x3 |
| Vm 15 | 0x1 | 0x4 |
| Vp 12 | 0x1 | 0x5 |
| Vm 12 | 0x1 | 0x6 |
| Vcc 5 | 0x1 | 0x7 |
| Vcc 5 Analog | 0x1 | 0x8 |
| Vp 4 | 0x1 | 0x9 |
| Vm 3 | 0x1 | 0xA |
| Vm 2 | 0x1 | 0xB |
| Vee | 0x1 | 0xC |
| Vref 5 | 0x1 | 0xD |
| Vpref 3 | 0x1 | 0xE |
| Vmref 3 | 0x1 | 0xF |
| Not used (Disable) | 0x1 | 0x10 to 0xFFFF |

Use shift registers for addressing  multiplexes: MM74HC595 . Registers are chained by 2 : 2 registers for mux address to be visualised, 2 registers to validate the channels 0 to3 visualisation and 2 registers to validate the channel 4 to 7 visualisation. There is one Mux Clock signal for all registers and one Strobe Mux for one group of two registers :

Mux Strobe 0 : Validation of channel 4 to 7 visualisation

Mux Strobe 1 : Validation of channel 0 to 3 visualisation

Mux Strobe 2 : Mux address to be visualise.

MUX Data are sent per 16 bits MSB first : Q[7:0]1-Q[7:0]2

| Mux Strobe | Mux Clock | Global Reset |  |
| --- | --- | --- | --- |
| X | X | L | Clear register, serial out=0 |
| X |  | H | Qn=Qn-1, Qo=serial input |
|  | X | H | Contents of register transfered to output |

| Mux Strobe 2 | Mux Strobe 1 | Mux Strobe 0 | Logic and analogic lines |  | Card Channel number |
| --- | --- | --- | --- | --- | --- |
|  |  |  | address | number |  |
| 0 | 0 | 1 | 0x20, 0x28 | 1 | 0 |
| 0 | 0 | 1 | 0x20, 0x28 | 1 | 1 |
| 0 | 0 | 1 | 0x20, 0x28 | 1 | 2 |
| 0 | 0 | 1 | 0x20, 0x28 | 1 | 3 |
| 0 | 1 | 0 | 0x20, 0x28 | 1 | 4 |
| 0 | 1 | 0 | 0x20, 0x28 | 1 | 5 |
| 0 | 1 | 0 | 0x20, 0x28 | 1 | 6 |
| 0 | 1 | 0 | 0x20, 0x28 | 1 | 7 |
| 0 | 0 | 1 | 0x24, 0x2C | 2 | 0 |
| 0 | 0 | 1 | 0x24, 0x2C | 2 | 1 |
| 0 | 0 | 1 | 0x24, 0x2C | 2 | 2 |
| 0 | 0 | 1 | 0x24, 0x2C | 2 | 3 |
| 0 | 1 | 0 | 0x24, 0x2C | 2 | 4 |
| 0 | 1 | 0 | 0x24, 0x2C | 2 | 5 |
| 0 | 1 | 0 | 0x24, 0x2C | 2 | 6 |
| 0 | 1 | 0 | 0x24, 0x2C | 2 | 7 |

Mux Strobe 2 is asserted by writing parameter data value for logic, analogic and voltage lines. In this case, Mux Strobe 0 and 1 must be deasserted.

Data format is :

| Q7-1 | Q6-1 | Q5-1 | Q4-1 | Q3-1 | Q2-1 | Q1-1 | Q0-1 | Q7-2 | Q6-2 | Q5-2 | Q4-2 | Q3-2 | Q2-2 | Q1-2 | Q0-2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parv |  |  |  | Parl2 |  |  | Parl1 |  |  | Para2 |  |  | Para1 |  |  |

#### Temperature Control Register

VME base Address :  0x40

| Address | Function | Command |
| --- | --- | --- |
| 0x40 | Read Temperature | 0xAA |
| 0x42 | Write Temperature High | 0x1 |
| 0x44 | Write Temperature High | 0x2 |
| 0x46 | Read Temperature High | 0xA1 |
| 0x48 | Read Temperature Low | 0xA2 |
| 0x4A | Read Counter | 0xA0 |
| 0x4C | Read Slope | 0xA9 |
| 0x4E | Start Convert Temperature | 0xEE |
| 0x50 | Stop Convert Temperature | 0x22 |
| 0x52 | Write Config | 0xC |
| 0x54 | Read Config | 0xAC |

#### GE Card EEPROM

VME Base Address : 0x100 to 0x1FC

##### Ge Card  Identification (Analogue part):

VME Base Address : 0x100 to 0x10A

| VME Base Address | Description | DATA Format |
| --- | --- | --- |
| 0x100 | In Read Mode this is the Man. ID | D7..D0 from XCPROM |
| 0x102 |  |  |
| 0x104 |  |  |
| 0x106 |  |  |
| 0x108 | Read Only: Serial no. | Read 8 bits only from XCPROM d7 to d0 |
| 0x10a | Read Only: Modification Level | Read 8 bits only from XCPROM d7 to d0 |

##### DAC Sliding Scale Value

VME Base Address : 0x110 to 0x17E

| VME Base Address | Description | DATA Format |
| --- | --- | --- |
| 0x110 | DAC Sliding Scale Ramp Value ADC0 ch0 |  |
| 0x112 | DAC Sliding Scale Offset Value ADC0 ch0 |  |
| 0x114 | DAC Sliding Scale Ramp Value ADC1 ch0 |  |
| 0x116 | DAC Sliding Scale Offset Value ADC1 ch0 |  |
| 0x118 | DAC Sliding Scale Ramp Value ADC2 ch0 |  |
| 0x11A | DAC Sliding Scale Offset Value ADC2 ch0 |  |
| ... |  |  |
| 0x12A | DAC Sliding Scale Ramp Value ADC2 ch7 |  |
| 0x13C | DAC Sliding Scale Offset Value ADC2 ch7 |  |

#### E.V.R.I. Readout Memory Map :

VME base Address : 0x300 to 0x3FC.

| VME Base Addresses | Bus Width | Read/ Write | Function |
| --- | --- | --- | --- |
| 0x300 | 16 | r/w | Event Counter Register |
| 0x302 | 16 | r/w | Item + Group no. for the event counter Register |
| 0x300 | 32 | read only | Item + Group no. + Event Counter |
| 0x304 | 32 | r/w | Fifo Test Access |
| 0x308 | 16 | r/w | DSP Control Register (see §2.2.2.6.1) |
| 0x30A | 16 | r/w | EVRI Readout Control Register(see §2.2.2.6.2) |
| 0x30C | 16 |  | Readout Status Register(see §2.2.2.6.3) |
| 0x30E | 16 | r/w | Valack Timeout Register |
| 0x310 | 16 | read only | Oe Qbus Counter |
| 0x312 | 16 | read only | Start Paril Test |
| 0x314 | 16 | read only | Stop Paril Test |
| 0x316 to 0x3FC | N.A. | r/w | Not yet Implemented |

##### DSP Control Register (DCR)

VME base Address : 0x308

| Bit Number | Meaning when = 0 | Meaning when = 1 |
| --- | --- | --- |
| 0 | Datas Transfert between PARIL and DSP disable | Datas Transfert between PARIL and DSP enable |
| 1 | Datas Transfert between DSP and VXI bus disable | Datas Transfert between DSP and VXI bus enable |
| 2 | DSP Link Transfert Disable | DSP Link Transfert Enable |
| 3 | DMA Test Disable | DMA Test Enable |
| 4 to 15 | Not yet defined | Not yet defined |

- Note : Bit 0 and bit 1 are valid only if the DSP Link Transfert is enable (bit 2 =1)

##### E.V.R.I. Readout Control Register (RCR)

VME base Address : 0x30A

| Bit Number | Meaning when = 0 | Meaning when = 1 |
| --- | --- | --- |
| 0 | Disable Valack Timeout Reset | Enable Valack Timeout Reset |
| 1 | Enable Readout from E.V.R.I. | Disable Readout from E.V.R.I. (card is bypass) |
| 2 | Event Number Readout ON | Event Number Readout OFF |
| 3 | Qstat Disable | Qstat Enable |
| 4 | Pipeline Readout Disable | Pipeline Readout Enable |
| 5 | Common DeadTime Readout Disable | Common DeadTime Readout Enable |
| 6 | External Readout Disable | External Readout Enable |
| 7 | Test Mode Paril Disable | Test Mode Paril Enable |
| 8 to 15 | Not yet defined | Not yet defined |

##### E.V.R.I. Readout Status Register

VME base Address :0x30C

| Bit Number | Meaning when = 0 | Meaning when = 1 |
| --- | --- | --- |
| 0 | E.V.R.I. Temperature indicator is above the upper threshold | E.V.R.I. Temperature indicator is below the upper threshold |
| 1 | E.V.R.I. Temperature indicator is below the lower threshold | E.V.R.I. Temperature indicator is above the lower threshold |
| 2 | Temperature indicator 1 is above the upper threshold | Temperature indicator 1 is below the upper threshold |
| 3 | Temperature indicator 1 is below the lower threshold | Temperature indicator 1 is above the lower threshold |
| 4 | Temperature indicator 2 is above the upper threshold | Temperature indicator 2 is below the upper threshold |
| 5 | Temperature indicator 2 is below the lower threshold | Temperature indicator 2 is above the lower threshold |
| 6 | Valack Timeout deasserted | Valack Timeout asserted |
| 8 | Empty Flag of Fifo1 asserted | Empty Flag for Fifo1 deasserted |
| 9 | Full Flag of Fifo1 asserted | Full Flag for Fifo1 deasserted |
| 10 to 15 | not defined | not defined |

## Channel Area Addresses

| VME ADDRESS | Bus Width | Read/ Write | Function |
| --- | --- | --- | --- |
| Channel 0 (0x1000 --0x1FFE) |  |  |  |
| 0x1000 to 0x100E | 16 | w | Dac Adjustment for channel 0 (see §2.3.1) |
| 0x1010 to 0x101E | 16 | r/w | Potentiometer Adjustment for channel 0 (see § 2.3.2) |
| 0x1020 | 16 | w | Channel Control Register (see §2.3.3) |
| 0x1030 | 16 | w | DAC output mode select for channel 0 (see §2.3.4) |
| 0x1200 to 0x12FC | 32 | r/w | PARIL Setup Channel 0 (see §2.3.5) |
| Channel 1 (0x2000 --0x2FFE) |  |  |  |
| 0x2000 to 0x200E | 16 | w | Dac Adjustment for channel 1 |
| 0x2010 to 0x201E | 16 | w | Potentiometer Adjustment for channel 1 |
| 0x2020 | 16 | r/w | Channel Control Register |
| 0x2030 | 16 | w | DAC output mode select for channel 1 |
| 0x2200 to 0x22FC | 32 | r/w | PARIL Setup Channel 1 |
| Channel 7 (0x8000 --0x8FFE) |  |  |  |
| 0x8000 to 0x800E | 16 | w | Dac Adjustment for channel 7 |
| 0x8010 to 0x801E | 16 | w | Potentiometer Adjustment for channel 7 |
| 0x8020 | 16 | r/w | Channel Control Register |
| 0x8030 | 16 | w | DAC output mode select for channel 7 |
| 0x8200 to 0x82FC | 32 | r/w | PARIL Setup Channel 7 |

### DAC Adjustment.

VME Base Address = 0x1000 +n*0x1000 (n=0 to 7)

- MAX528 is an octuple serial 8 bits DAC, with buffered output. MSB sent first.

- Exemple given for channel 0.

| DAC 0 (0x1000 --0x10FE) | DAC Parameter Name |
| --- | --- |
| 0x1000 | PDS Gate Width Channel 0 |
| 0x1002 | PZ Threshold Adjust Channel 0 |
| 0x1004 | 6 MeV Offset Channel 0 |
| 0x1006 | 20 MeV Offset Channel 0 |
| 0x1008 | FT Sample Threshold Channel 0 |
| 0x100a | CFD Output DELAY Channel 0 |
| 0x100c | Pileup Threshold Channel 0 |
| 0x100e | CFD Threshold channel 0 |

To load DAC values, Data format is :

- A0-A7 assigned to Out0-Out7

D0-D7 : DAC value written to the output register for witch the address bit is asserted.

If it is not, the previous value is maintained.

The output voltage will be : Vout = (RefH-RefL)*(data/256)+RefL.

### Potentiometer  Memory Map:

VME Base Address = 0x1010 +n*0x1000 (n=0 to 7)

| Potar 0 (0x1010 --0x101E) | Potar Parameter Name |
| --- | --- |
| 0x1010 | 6 Mev Gain – ve Adjustment |
| 0x1012 | 6 Mev Gain + ve Adjustment |

AD8402 is a double digital potentiometer. Data are sent MSB first 1:

- Bits 0 to 7 : data

Bits 9,8 : 00 negative adjustment, 01 positive adjustment (address).

Each bit is latched with the rising edge of the clock during CS* is active low.

### CCR Definition

VME Address 0x1020 +n*0x1000 (n=0 to 7)

| Bit Number | Meaning when = 0 | Meaning when = 1 |
| --- | --- | --- |
| 0 | Disable Channel | Enable Channel |
| 1 | Disable Test Gene | Enable Test Gene |
| 2 | Negative PA Input Polarity | Positive PA Input Polarity |
| 3 | Disable DPPC | Enable DPPC |
| 4 | Range 20 MeV Shaping Amplifier | Range 3 MeV Shaping Amplifier |
| 5 | DPPC Charge Input | DPPC Current Input |
| 6 | TAC Range 0 to 200 ns | TAC Range 0 to 2µs |
| 7 to 15 | Not Allocated | Not Allocated |

- MM74HC595 : 8 bits Shift register.  1

One register per channel. Datas are sent MSB first. One CCR Clock signal for all channel, and one CCR Strobe per channel.

| CCR Strobe | CCR Clock | Global Reset |  |
| --- | --- | --- | --- |
| X | X | L | Clear register, serial out=0 |
| X |  | H | Qn=Qn-1, Qo=serial input |
|  | X | H | Contents of register transfered to output |

### DAC Output Mode Programmation

VME Base Address = 0x1030 +n*0x1000 (n=0 to 7)

For the output mode programmation (in our case, all outputs are buffered), the following word must be sent MSB First : 0xFF . See below the details and signification of this word.

| A7 | … | A0 | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | X | 1 | 1 | 1 | 1 | 1 | 1 |
| Buffered Outputs Mode Select |  |  |  |  | Out 0&1 | Out 2&3 | Out 0-3 | Out 4&5 | Out 6&7 | Out 4-7 |
|  |  |  |  |  | RefH = +3V; RefL = -3V |  |  | RefH = +5V; RefL = 0V |  |  |

### PARIL Setup.

VME Base Address : 0x1200 +n*0x1000 (n=0 to 7)

| VME Base Addresse | Bus Width | Read/ Write | Function |
| --- | --- | --- | --- |
| 0x1200 to 0x123C | 32 | r/w | Corrolated Event Datas Memory (see § 2.3.5.1) |
| 0x1240 | 32 | r/w | Number of DPPC datas to be readout |
| 0x1244 | 32 | r/w | DPPC Readout Memory Offset Address |
| 0x1248 | 32 | r/w | Time frame Generator Counter register |
| 0x124C | 32 | r/w | Time frame Generator Step |
| 0x1250 | 16 | r/w | PARIL Corrolated Control Register (see § 2.3.5.2) |
| 0x1254 | 16 | r/w | PARIL Singles Control Register (see § 2.3.5.3) |
| 0x1258 | 32 | r/w | PARIL Inspection Line 1 (see § 2.3.5.4) |
| 0x125C | 32 | r/w | PARIL Inspection Line 2 |
| 0x1260 | 32 | r/w | Simulate Start Readout Single Command |
| 0x1264 | 32 | r/w | Simulate Start Readout Corrolated Command |
| 0x1268 | 32 | r/w | Simulate End Of Convert to DPPC |
| 0x126C | 32 | r/w | Simulate Encode Command from LT to ADC |
| 0x1270 | 32 | r/w | Simulate End-DPPC-Readout |
| 0x1280 | 16 | r/w | Read/Write VME DPPC Datas (at offset specified below) |
| 0x1284 | 16 | wo | Write Offset Address in DPPC to read/write datas by vme |
| 0x1288 | 16 | ro | Read Scaler |
| 0x128C | 16 | ro | Real TFG Count |

#### Corrolated Event Datas Memory

VME Base Address = 0x1200 +n*0x1000 (n=0 to 7)

| 0x1200 | ADC 6 Mev Data |
| --- | --- |
| 0x1204 | ADC 20 Mev Data |
| 0x1208 | TAC Data |
| 0x120C to 0x123C | Pulse Processing Results (see DPPC doc) |

#### PARIL Corrolated Control Register (PRCR)

VME Base Address = 0x1250 +n*0x1000 (n=0 to 7)

| Bit Number | Meaning when = 0 | Meaning when = 1 |
| --- | --- | --- |
| 0 to 2 | Disable Corrolated Event Readout in Experiment Mode | Enable Corrolated Event Readout in Experiment Mode (see § 2.3.5.2.1) |
| 3 to 15 | Disable DPPC Readout in Experiment Mode | Enable DPPC Readout in Experiment Mode |

##### Corrolated Event Readout in Experiment Mode

| Bit Number | Meaning when = 0 | Meaning when = 1 |
| --- | --- | --- |
| 0 | Disable ADC 0 Readout (20 MeV). | Enable ADC 0 Readout(20 MeV). |
| 1 | Disable ADC 1 Readout(6 MeV). | Enable ADC 1 Readout(6 MeV). |
| 2 | Disable ADC 2 Readout(TAC). | Enable ADC 2 Readout(TAC). |

#### PARIL Singles Control Register (PRCS)

VME Base Address = 0x1254 +n*0x1000 (n=0 to 7)

| Bit Number | Meaning when = 0 | Meaning when = 1 |
| --- | --- | --- |
| 0 | Disable Singles Readout | Enable Singles Readout |
| 1 | Enable TAC | Enable TFG |
| 2 | Disable TFG to count | Enable TFG to count |
| 3 | Enable Link_Ack to Front Panel | Enable Link_Ack to EVRI |
| 4 | Disable LT Reset | Enable LT Reset |

#### PARIL Inspection Lines 1 et 2

Vme Address line 1 : 0x1258 + 0x1000*n (n=0 to 7)

Vme Address line 2 : 0x125C + 0x1000*n (n=0 to 7)

| PARIL Signals | Parameter Value | PARIL Signals | Parameter Value |
| --- | --- | --- | --- |
| Disable | 0x000 | CS-Ram-Run | 0x10 |
| Encod | 0x01 | End_DPPC_Readout | 0x11 |
| Single-Ack | 0x02 | WS-6Mev-Run | 0x12 |
| Read-Convert | 0x03 | WS-TAC-Run | 0x13 |
| Busy | 0x04 | CS-Singles-Run | 0x14 |
| End-of-Convert | 0x05 | Prepileup | 0x15 |
| Data-Valid-Dppc | 0x06 | Postpileup | 0x16 |
| CDR | 0x07 | Veto | 0x17 |
| Dppc-IAL | 0x08 | Start Singles Readout | 0x18 |
| Dppc-CS | 0x09 | Latch Singles | 0x19 |
| Dppc-Wr | 0x0A | Link Ack | 0x1A |
| Dppc-Rd | 0x0B | Oe Link Bus Evt | 0x1B |
| Iack | 0x0C | Link Bus In 0 | 0x1C |
| Dppc-Add-Run | 0x0D | Link Clk | 0x1D |
| Corr-Add-Run | 0x0E | Count TFG | 0x1E |
| WS-Ram-Run | 0x0F | Clear TFG | 0x1F |

## SRAM Area Address

VME address : 0x20 0000 to 0x3F FFFC

| SRAM Address | VME Address | Spectra |
| --- | --- | --- |
| 0x0+(ch nb)*0xC000 | 0x20 0000+(ch nb)*0x3 0000 | ADC 6 Mev Data |
| 0x4000+(ch nb)*0xC000 | 0x21 0000+(ch nb)*0x3 0000 | ADC 20 Mev Data |
| 0x8000+(ch nb)*0xC000 | 0x22 0000+(ch nb)*0x3 0000 | TAC Data |
| 0x6 0000 | 0x38 0000 | Echelles |

Ch nb : Channel Number = 0 to 7.

Spectra are coded on 14 bits (0 to 13) when HRGe8x.ldr DSP program is used.

Example given for channel 0 and channel 7.

| SRAM Address | VME Address | Spectra |
| --- | --- | --- |
| 0x0 | 0x20 0000 | ADC 6 Mev for channel 0 |
| 0x4000 | 0x21 0000 | ADC 20 Mev for channel 0 |
| 0x8000 | 0x22 0000 | TAC for channel 0 |
| 0x5 4000 | 0x35 0000 | ADC 6 Mev for channel 7 |
| 0x5 8000 | 0x36 0000 | ADC 20 Mev for channel 7 |
| 0x5 C000 | 0x37 0000 | TAC for channel 7 |

## DSP Area Address

VME address : 0x40 0000 to 0x4B FFFC

| DSP Address | VME Address | Data |
| --- | --- | --- |
| 0x0 to 0xFF | 0x40 0000 to 0x40 03FC | IOP Registers (See §2.5.1) |
| 0x2 0004 to 0x2 00FF | 0x48 0010 to 0x48 03FC | Interrupt Vectors |
| 0x2 0100 to 0x2 2FFF | 0x48 0400 to 0x48 BFFC | Subroutines |
| 0x2 3000 to 0x2 5FFF | 0x48 C000 to 0x49 7FFC | Data 48 bits |
| 0x2 6000 to 0x2 60FF | 0x49 8000 to 0x49 83FC | Variables (See §2.5.2) |
| 0x2 6100 to 0x2 FFFF | 0x49 8400 to 0x4B FFFC | Data 32 bits (See §) |

### IOP Registers

VME address : 0x40 0000 to 0x40 03FC

| DSP Address | VME Address | Data |
| --- | --- | --- |
| 0x0 | 0x40 0000 | Syscon Register |
| 0x2 | 0x40 0008 | Wait Register |
| 0x1D | 0x40 0074 | DMA7 Control Register |
| 0x48 to 0x4F | 0x40 0120 to 0x40 013C | DMA Channel 7 Registers (See § |
| 0xC0 to 0xC8 | 0x40 0300 to 0x40 0320 | Link Ports Registers (See § |

#### DMA Channel 7Registers

VME address : 0x40 0120 to 0x40 013C

| DSP Address | VME Address | Data |
| --- | --- | --- |
| 0x48 | 0x40 0120 | II7  Internal memory address |
| 0x49 | 0x40 0124 | IM7  Internal memory access modifier |
| 0x4A | 0x40 0128 | C7  Number of transfers remaining |
| 0x4D | 0x40 0134 | EI7  External memory address |
| 0x4E | 0x40 0138 | EM7  External memory access modifier |
| 0x4F | 0x49 80140x40 013C | EC7  External Counter |

#### Link Ports Registers

VME address : 0x40 0300 to 0x40 0320

| DSP Address | VME Address | Data |
| --- | --- | --- |
| 0xC0 | 0x40 0300 | LBUF0  Link buffer 0 |
| 0xC1 | 0x40 0304 | LBUF1  Link buffer 1 |
| 0xC2 | 0x40 0308 | LBUF2  Link buffer 2 |
| 0xC3 | 0x40 030C | LBUF3  Link buffer 3 |
| 0xC4 | 0x40 0310 | LBUF4  Link buffer 4 |
| 0xC5 | 0x40 0314 | LBUF5  Link buffer 5 |
| 0xC6 | 0x40 0318 | LCTL  Link buffer Control |
| 0xC7 | 0x40 031C | LCOM  Link Common control |
| 0xC8 | 0x40 0320 | LAR  Link Assignment register |

### Variables

VME address : 0x49 8000 to 0x49 83FC

| DSP Address | VME Address | Data |
| --- | --- | --- |
| 0x26000--0x2601F | 0x49 8000--0x49 807C | Spectra and Scaler (see §2.5.2.1) |
| 0x26020—0x2602F | 0x49 8080--0x49 80BC | Power Supplies Inspection (see §2.5.2.2) |

#### Spectra and Scaler

| DSP Address | VME Address | Data |
| --- | --- | --- |
| 0x26000 | 0x49 8000 | Subroutine number |
| 0x26001 | 0x49 8004 | First Item number |
| 0x26002 | 0x49 8008 | Event number |
| 0x26003 | 0x49 800C | Data number per event |
| 0x26004 | 0x49 8010 | Error counter on item number |
| 0x26005 | 0x49 8014 | Error counter on data number |

#### Power Supplies Inspection

| DSP Address | VME Address | Data |
| --- | --- | --- |
| 0x26020 | 0x49 8080 | Multiplexer Input Number |
| 0x26021 | 0x49 8084 | Coding Resolution |

### Data 32 bits

VME address : 0x49 8400 to 0x4B FFFC

| DSP Address | VME Address | Data |
| --- | --- | --- |
| 0x26100--0x261FF | 0x49 8400—0x49 87FC | Spectra and Scaler (see §2.5.3.1) |
| 0x26200--0x266FF | 0x49 8800--0x49 9BFC | Power Supplies Inspection (see §2.5.3.2) |

#### Spectra and Scaler

| DSP Address | VME Address | Data |
| --- | --- | --- |
| 0x26100 + (ch nb)*0x3 | 0x49 8400 + (ch nb)*0xC | ADC 6 Mev Data |
| 0x26101 + (ch nb)*0x3 | 0x49 8404 + (ch nb)*0xC | ADC 20 Mev Data |
| 0x26102 + (ch nb)*0x3 | 0x49 8408 + (ch nb)*0xC | TAC Data |

Ch nb : Channel Number = 0 to 7.

Example given for channel 0 and channel 7.

| DSP Address | VME Address | Data |
| --- | --- | --- |
| 0x26100 | 0x49 8000 | ADC 6 Mev Data for channel 0 |
| 0x26101 | 0x49 8004 | ADC 20 Mev Data for channel 0 |
| 0x26102 | 0x49 8008 | TAC Data for channel 0 |
| 0x26103 | 0x49 800C | ADC 6 Mev Data for channel 7 |
| 0x26104 | 0x49 8010 | ADC 20 Mev Data for channel 7 |
| 0x26105 | 0x49 8014 | TAC Data for channel 7 |

#### Power Supplies Inspection.

| DSP Address | VME Address | Data |
| --- | --- | --- |
| 0x26200—0x26206 | 0x49 8800 | Multiplexer input selection |
| 0x26207—0x26214 | 0x49 881C | Power Supplies Table Reference |
| 0x26215—0x2621B | 0x49 8854 | Power Supplies Average |
| 0x2621C—0x26222 | 0x49 8870 | Deviation |
| 0x26223—0x26622 | 0x49 888C | ADC Data |

# general figures

## EVRI Card Dimensions.

## PCB_AP board connections:

### EVRI version.

GENERAL INTERFACE READOUT

ANALOG FRONT END

5.00 mm

Ref : Advanced KMA-030-165TG

Ref : Advanced KMS-030-95TG

110.08 mm

2 mm

### GIR V5 Version.

110.08 mm

GENERAL INTERFACE READOUT

ANALOG FRONT END

5.00 mm

Ref : Advanced KMA-030-165TG

Ref : Advanced KMS-030-95TG

2 mm

## HRGe8x Shift registers connection figure

Q0-1

Q0-2

Serial In

Reg1

Reg2

Serial In

Q7-1

Q7-2

Mux Clock

Mux Clock

Serial Out

Serial Out

Mux Strobe n

Mux Strobe n

## HRGe8x Dac and potentiometer connection figure

DAC n

DAC n+1

Potar n

Potar n+1

AP1

Connector

Dac Select

Dac Data

DCSn

DCSn+1

Pot Select

Dac Clk

See the VXI Lines Definition Document by: A. Richard

See the Euroball Trigger Specification by: I. Lazarus CLRC,, Daresbury U.K.

See the Eurogam VXI Readout Specification by J. Alexander.

Bit 9 = 0/1 : enable/disable analogue Sysclk in read mode

Bit 2 = 0/1 : enable/disable analogue  Sysclk in write mode

See Procedure de reglage des parametres document for more details (V. Hervier).

In addback Mode, no singles events.

See Procedure de reglage des parametres document for more details (V. Hervier).

set bit 6 of GICR register to write the values. See §

(multiple of 31.25 ns) (100 <t< 4 sec.) (active when bit0 of RCR is 1)

e.g. to have 100 us timeout write the value 0xC80 (3200).

See Procedure de reglage des parametres document for more details (V. Hervier).

- To visualise Paril Inspection Lines, select on digital line 1 or 2

Channel=0x20, Parameter=0x36 for Paril line 1 or 0x37 for Paril line2